Zeiss Axiovert 200 MAT w/ Axiocam MRc5 microscope

Standard Operation Procedure

Microscope Layout

Stage

Manual adjustment knob: Forward/Back Left/Right

LCD Display

Halogen Lamp on/off switch

Toggle switch for illumination intensity

Course/Fine focusing knobs

On/Off Switch

Revolving objective nosepiece

Imaging mode selector:              BF

DF

DIC

Optovar turret selector wheel: 1x or 1.5x

Selector wheel for camera port: Microscope and computer, Microscope only, Computer only.

- Microscope Basic Operation: Microscope View ONLY

  - Power microscope on using the On/Off Switch

  - Verify that the Selector wheel for camera port is set to Microscope Only

  - Check that light is coming from the microscope up through the stage

  - Selective desired objective nosepiece. Note: It is usually best to start with the lowest and then increase the objective nosepiece after focusing

  - Place specimen on sample stage in region of light

  - Use the Manual adjustment knob and Course/Fine focusing knob to find and focus on region of interest.

  - Image brightness can be adjusted using the illumination intensity toggle switch.

  - For more brightness and image adjustment options, see complete user manual located in a drawer below microscope.

- Microscope Basic Operation: Computer View and/or Microscope

  - Power microscope on using the On/Off Switch

  - Check that light is coming from the microscope up through the stage

  - Selective desired objective nosepiece. Note: It is usually best to start with the lowest and then increase the objective nosepiece after focusing

  - Place specimen on sample stage in region of light

  - Use the Manual adjustment knob and Course/Fine focusing knob to find and focus on region of interest.

  - Image brightness can be adjusted using the illumination intensity toggle switch.

  - Verify that the Selector wheel for camera port is set to Computer Only

or Microscope and Computer

  - Open AxioVision SE64 Rel. 4.8 Software

  - Click Live on the toolbar

  - Use the Manual adjustment knobs to move

area of interest into computer screen view

  - Select Snap to capture image

  - Scale bar and text can be added using the software

  - Save image

If you have any questions, ask for assistance or consult complete operating manual located in a drawer below the microscope.